TV / COMPUTER CART

Streaming Station, Sanctuary Lobby

TURNING ON

| 1 | Turn on the computer. Green button just left of the USB port on the front of the PC. |
| --- | --- |

| 2 | Turn on the TV. Remote power button, or the top button on the TV itself. |
| --- | --- |

| 3 | Log in. Touch the keyboard or mouse to wake the screen. Ask this week’s Hospitality team tech volunteer for the PIN if you don’t have it. |
| --- | --- |

| 4 | Open Zoom Workplace and start/join the meeting. Already logged into the church account, so no separate sign-in needed. |
| --- | --- |

VERIFY THESE ARE SET (should stay set once configured)

| Camera | Logi Rally Bar Mini |
| --- | --- |
| Microphone | iRig Pro |
| Speaker | Rally Bar Mini |

HEARING ECHO OR FEEDBACK ON THE STREAM? • "Original sound for musicians" should be ON • "Echo cancellation" should be OFF • "Automatically adjust microphone volume" should be OFF • Zoom updates sometimes silently reset these. Check here first if the stream sounds off.

TALKING TO PEOPLE ON ZOOM • Small mic on the cart. Press and HOLD the center button to talk. • Light comes on while you're holding it. Let go and it turns off automatically.

Scan for the full guide + troubleshooting